TRIANGLE WAVEFORM GENERATION WITH 50% DUTY CYCLE USING 10 BIT DAC

AIM:

To write the C program to generate a triangular wave form with 50% duty cycle using

internal 10-bit DAC using LPC2148 ARM Micro controller.

APPARATUS REQUIRED:

- LPC 2148 ARM Microcontroller Development board.

- Keil µVision version 5

- Flash Magic

PROGRAM:

#include <lpc214x.h>

void delay(unsigned int count);      // Function for generating a delay

void generate_triangle_wave(void);   // Function to generate triangular waveform

int main(void) {

// Initialize DAC on P0.25

PINSEL1 |= (1 << 19);   // Configure P0.25 as DAC output

while (1) {

generate_triangle_wave();    // Generate triangular wave

delay(50000);                // Small delay between waveform switching

}

}

void delay(unsigned int count) {

unsigned int i, j;

for (i = 0; i < count; i++) {

for (j = 0; j < 6000; j++);  // Approximate delay

}

}

// Function to generate triangular waveform using DAC

void generate_triangle_wave(void) {

unsigned int i;

// Incrementing part of the triangle

for (i = 0; i < 1023; i++) {

DACR = (i << 6);  // Write to DAC (10-bit left justified)

delay(100);       // Delay to control waveform frequency

}

// Decrementing part of the triangle

for (i = 1023; i > 0; i--) {

DACR = (i << 6);  // Write to DAC

delay(100);       // Delay to control waveform frequency

}

}